Assignment 1: Branch Predictor Implementation

Akhil P

1. Goal

The goal of this assignment was to implement the branch direction prediction part of the Pentium M branch predictor in C++ and to evaluate it on real program traces using the CBP2 simulation infrastructure. The implementation combines a bimodal predictor with a 2-way tagged global predictor. The loop predictor and branch target prediction were skipped, as allowed by the assignment.

2. Background

A branch predictor guesses whether a branch (such as an if or a loop condition) will be taken or not taken before the processor knows the real outcome. A wrong guess forces the pipeline to throw away work, so better prediction means better performance and lower power.

- 2-bit saturating counter: a counter that goes from 0 to 3. Values 0 and 1 predict not taken, and values 2 and 3 predict taken. After each branch it moves up if the branch was taken and down if it was not, and it stops at 0 and 3.

- MPKI: mispredictions per 1000 instructions. This is the metric used throughout this report, and lower is better.

- Global history register (GHR): a shift register holding the outcomes of the most recent branches (1 = taken, 0 = not taken).

3. Design

The predictor (pm_predictor in src/my_predictor.h) has two parts that work together.

Bimodal predictor

- 4096 2-bit counters, indexed by the low 12 bits of the branch address.

- It captures how a given branch usually behaves, but it has no knowledge of what happened just before the branch.

- Counters start at 1 (weakly not taken).

Global predictor

- 512 sets, 2 ways per set (a two-way global predictor, which the assignment allows instead of four-way). These sizes follow the Pentium M diagram in the assignment handout.

- Each entry holds a valid bit, a 6-bit tag and a 2-bit counter.

- The index is built by XORing the low 15 bits of the branch address with a 15-bit GHR. The top 9 bits of the result select the set, and the low 6 bits are the tag.

- Because the history is part of the index, the same branch can get different predictions depending on the recent outcomes, which lets the predictor learn patterns that depend on history.

How the two are combined

- If one of the two ways in the selected set has a matching tag (a global hit), the final prediction is that entry’s counter.

- If neither way matches (a global miss), the final prediction comes from the bimodal predictor.

- Unconditional branches are always predicted taken.

4. Implementation Details

- Update: after each conditional branch resolves, the bimodal counter is always updated. On a global hit, the matching counter is updated and that way is marked most recently used. On a global miss, a new entry is created in an empty way, or in the least recently used way if both are occupied (LRU replacement). The new counter starts at 2 if the branch was taken and 1 if it was not taken.

- History: the GHR is shifted left by one and the new outcome is inserted after every conditional branch.

- Reused code: the structure of the provided gshare_predictor class (the XOR of history and address, and the 2-bit counter update) was used as the starting point. No external cache code was used.

- Parameters: bimodal index 12 bits, history and hash 15 bits, tag 6 bits, 512 sets, 2 ways.

- Skipped: loop predictor, branch target prediction, and the PIR with its hash, as permitted by the assignment.

5. How to Build and Run

cd src && make && cd ..

./src/predict traces/164.gzip/gzip.trace.bz2   # one trace

bash runall.sh      # all traces + average MPKI

runall.sh runs the predictor on every trace and prints the MPKI for each one plus the average.

6. Results

Both versions were run on all 20 traces. “Bimodal only” is the same code with the global predictor ignored, so the comparison isolates the benefit of adding the global predictor.

| Trace | Bimodal only (MPKI) | Hybrid (MPKI) |
| --- | --- | --- |
| gzip | 15.030 | 12.617 |
| vpr | 14.671 | 14.337 |
| gcc | 19.347 | 15.298 |
| mcf | 33.245 | 17.680 |
| crafty | 13.075 | 8.464 |
| parser | 15.417 | 10.357 |
| compress | 9.356 | 8.199 |
| jess | 8.809 | 1.750 |
| raytrace | 3.806 | 3.421 |
| db | 6.480 | 4.613 |
| javac | 3.181 | 2.638 |
| mpegaudio | 3.154 | 2.604 |
| mtrt | 3.706 | 3.268 |
| jack | 7.691 | 3.946 |
| eon | 9.405 | 3.139 |
| perlbmk | 5.955 | 4.700 |
| gap | 9.675 | 4.797 |
| vortex | 1.916 | 1.239 |
| bzip2 | 0.112 | 0.076 |
| twolf | 21.942 | 22.046 |
| Average | 10.299 | 7.259 |

The hybrid predictor reduces the average MPKI from 10.299 to 7.259, an improvement of about 30%.

7. Analysis

Where the hybrid helps. The hybrid is better than bimodal alone on 19 of the 20 traces. The largest gains are on programs whose branches depend on recent history: mcf drops from 33.245 to 17.680 MPKI, jess from 8.809 to 1.750, eon from 9.405 to 3.139 and gap from 9.675 to 4.797. A bimodal predictor can only learn a branch’s overall bias, while the global predictor can learn that a branch behaves differently depending on the branches before it.

Where it does not help. twolf is the only trace where the hybrid is slightly worse (22.046 vs 21.942 MPKI). Both predictors perform poorly on it, so its branches are hard to predict from recent history; the exact cause was not investigated.

Effect of the table sizes. An earlier version of this predictor used the small sizes from the assignment’s worked example (256 sets, 4-bit tags, 12-bit hash). It averaged 9.337 MPKI and was worse than bimodal on 9 of the 20 traces. Switching to the sizes in the assignment’s Pentium M diagram (512 sets, 6-bit tags, 15-bit hash) lowered the average to 7.259. The likely reasons were not tested one at a time:

- Longer tags (6 bits instead of 4) mean fewer false hits, where two different branches or histories share one entry.

- Twice as many sets means fewer useful entries are evicted by newly allocated ones.

- A longer history (15 bits) lets the predictor separate more distinct branch contexts.

Possible improvements. Allocate a global entry only when the bimodal prediction was wrong, or use four ways as in the real Pentium M, and measure each change separately.

8. Conclusion

A hybrid predictor made of a 4096-entry bimodal table and a 2-way tagged global table (512 sets, 6-bit tags, 15-bit history) lowered the average MPKI from 10.299 to 7.259 across the 20 CBP2 traces, about 30%. It beat bimodal alone on 19 of 20 traces, with twolf the only exception.

9. References

- V. Uzelac and A. Milenkovic, “Experiment Flows and Microbenchmarks for Reverse Engineering of Branch Predictor Structures,” IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS), 2009.

- CBP2 (Championship Branch Prediction) simulation infrastructure, cbp2-infrastructure-v2, including the provided gshare example predictor.

- Course assignment handout, “Assignment 1: Branch Predictor Implementation.”